Basic Electric Concepts Exercise

| Author(s) | Gerard Elbers |
| --- | --- |
| Version | 0.1 |
| Date | 01-02-2026 |

# Document history

| Date | Version | Change |
| --- | --- | --- |
| 01-12-2025 | 0.1 | Initial version |
| 01-02-2026 | 1.0 | Introduction updated |

Inhoud

Document history	2

Introduction	4

Assignment	5

Part 1	5

Part 2	5

Part 3	6

# Introduction

With this exercise the student can demonstrate his proficiency in the Basic of Electronics  part of Learning Outcome 3:

- You understand the relationship between current, voltage and resistance and are able to make substantiated statements about basic electrical circuits with respect to those quantities using Ohm's Law and Kirchhoff's law.

- You can describe the electrical properties of LEDs and diodes

- You can calculate the equivalent resistance of series and parallel circuits of resistors

- You can lookup, derive and use the relevant properties of components from datasheets

# Assignment

## Part 1

Choose three different resistors and determine their values in three different ways:

- Using the colour codes

- Measuring the resistance with a multimeter

- Measuring the current through the resistor and the voltage over the resistor and using Ohm's law.

Make a table containing for each resistor the colours, the measured resistance, the current, the voltage and the calculated resistance. Include the table in the report

| Colour codes | Multimeter | Current |
| --- | --- | --- |
| Yellow, violet, red en gold = 4.7kohm | 4.67 kohm | 1.06 mA |
| Brown, red, brown en gold = 120 ohm | 118 ohms | 0.04 A |
| Orange, orange, brown en gold = 330 ohm | 325.6 ohms | 0.015 A |

## Part 2

Given the following simple network of resistors in parallel and series.

Figure 1 Simple resistor network

Calculate the equivalent resistor R13 of R1//R3, R123 of R13+R2 and R1234 of R123//R4.

Include the picture and your calculations in the report.

| R13 | 60 Ohm |
| --- | --- |
| R123 | 180 Ohm |
| R1234 | 72 Ohm |

R13 = R1 // R3 (parallel)

Twee weerstanden parallel: 1/R = 1/R1 + 1/R3

1/R13 = 1/120 + 1/120 = 2/120

R13 = 120/2 = 60 Ω

R123 = R13 + R2 (serie)

R123 = 60 + 120 = 180 Ω

R1234 = R123 // R4 (parallel)

1/R1234 = 1/R123 + 1/R4 = 1/180 + 1/120

Om dit op te tellen, gemeenschappelijke noemer = 360:

1/180 = 2/360 en 1/120 = 3/360

1/R1234 = 2/360 + 3/360 = 5/360

R1234 = 360/5 = 72 Ω

## Part 3

Given the following schematic and formula’s

Figure 2 LED with current limitting resistor

Use the data sheet of the LED to calculate the needed resitors for the RED, GREEN and YELLOW LEDs.

Use the typical values from the data sheet.

Include in the report:

- The given picture

- Snapshots of parts of the data sheet that you used

- The calculations

Why is the resistor needed? What would happen if you remove it?

To not let too much voltage trough the LED,

The LED would shine a bright light in its given color and then break after to be never able to be used again.

red
max
5-3 : 20 = 0.1 x 1000 =  100
typ
5-2 : 20 = 0.15 x 1000 = 150

green  
max
5-3 : 20 = 0.1 x 1000 =  100
typ
5-2.4 : 20 = 0.13 x 1000 = 130
yellow
max
5-3 : 20 = 0.1 x 1000 =  100
typ
5-2.4 : 20 = 0.13 x 1000 = 130

Submission Guidelines:

  - Students should submit a short report (1-5 pages) with answers to all questions.

  - Include calculations, graphs, or diagrams where applicable.